# Product Requirement Document (PRD)

# Smart Library Management & Tracking System (SLMTS)

Version: 1.0.0

Status: Approved for Implementation

Target Platform: Web Application (Responsive Desktop / Tablet / Mobile) + IoT Gate/Desk Edge Devices

# 1. Executive Summary & Vision

## 1.1 Problem Statement

Traditional libraries rely heavily on manual barcode scanning, physical registers, or legacy software (like raw Koha installations) that require significant manual intervention. Key pain points include:

Manual Circulation Delays: Staff scan barcodes one-by-one, leading to queues at peak hours.

Lack of Footfall & Time Analytics: Libraries record attendance manually or not at all; there is no real-time visibility into current occupancy or student dwell time.

Privacy & Cost Concerns with AI Cameras: Facial recognition systems raise severe privacy objections among students, suffer from lighting/angle failures, and require expensive GPU servers.

Book Misplacement & Unauthorized Removal: Optical barcodes require line-of-sight and cannot trigger automated anti-theft gate alarms reliably.

## 1.2 Product Vision

The Smart Library Management & Tracking System (SLMTS) delivers a modern, web-first Library Management System (LMS) inspired by Koha’s battle-tested circulation rules, but upgraded with:

Zero-AI, Privacy-Preserving Gate Tracking: Dual RFID/NFC card readers at gates to track entry, exit, live occupancy, and time spent.

Multi-Book HF RFID Circulation: Counter/kiosk pads capable of reading stacked books simultaneously without line-of-sight.

EAS Anti-Theft Gates: Automated security bit verification at exit points to stop unissued books.

# 2. User Personas & Roles

graph TD A[Super Admin / Head Librarian] -->¦Full Control, Policy & Reports¦ Core[SLMTS Platform] B[Circulation Desk Staff] -->¦Checkouts, Returns, Overrides¦ Core C[Students / Patrons] -->¦OPAC Search, Digital ID, Book Reservations¦ Core D[Security / Gate Hardware] -->¦Event-Driven In/Out Taps¦ Core

| Role | Access Level | Key Capabilities |
| --- | --- | --- |
| **Super Admin** | System-wide | Configure circulation rules, fine rates, branch libraries, hardware edge keys, role permissions. |
| **Librarian / Staff** | Operational | Catalog books, encode RFID tags, issue/return books, collect fines, view live student occupancy. |
| **Student / Patron** | Self-service | Search OPAC, view active loans, renew books, reserve titles, view personal library study hours. |
| **Hardware Service** | Machine-to-Machine | ESP32/Edge microcontrollers publishing gate scans and kiosk events via authenticated API/MQTT. |

# 3. Hardware Architecture (Non-Camera Approach)

sequenceDiagram autonumber actor Patron as Student participant GateIn as Gate Entry Reader (IN) participant Edge as ESP32 / Edge Controller participant Backend as LMS API Server participant GateOut as Gate Exit Reader (OUT) Note over Patron,GateIn: Entry Workflow Patron->>GateIn: Taps Smart ID Card (RFID/NFC) GateIn->>Edge: Transmits Card UID via Wiegand / SPI Edge->>Backend: POST /api/v1/gate/tap (UID, Direction=IN) Backend-->>Edge: HTTP 200 { status: 'GRANTED', patron_name: 'Khagendra' } Edge-->>GateIn: Beep (Green LED / Flap Unlock) Note over Patron,GateOut: Exit Workflow Patron->>GateOut: Taps Smart ID Card at Exit GateOut->>Edge: Transmits Card UID Edge->>Backend: POST /api/v1/gate/tap (UID, Direction=OUT) Backend-->>Edge: HTTP 200 { status: 'EXIT_RECORDED', duration: '1h 45m' } Edge-->>GateOut: Beep (Green LED)

## 3.1 Hardware Specifications

Patron Smart Card Reader (Gate In & Out):

Frequency: 13.56 MHz (Mifare Classic 1K / DesFire) or 125 kHz EM4100 / UHF.

Controller: ESP32-WROOM-32 with Ethernet/Wi-Fi + RC522 / PN532 or Wiegand-26/34 Industrial Wall Reader.

Feedback: Bi-color LED (Green/Red) and piezo buzzer (100ms chirp for grant, 3 beeps for deny).

Book Circulation RFID Reader (Desk / Kiosk):

Standard: ISO 15693 / ISO 28560 (Standard Library HF RFID).

Read Range: 15–25 cm (reads up to 5 books stacked on a flat pad).

Connection: USB HID / Virtual COM port to PC or WebUSB / Web Serial API directly inside Chrome/Edge.

Anti-Theft EAS Security Pedestal:

Installed at library exit gate. Scans ISO 15693 EAS/AFI security byte.

If bit is active (book not checked out), triggers audible 85dB alarm and red beacon strobe.

# 4. Functional Requirements

## Module 1: Cataloging & Book Management (Koha Inspired)

FR-1.1: Support standard book attributes: Title, Authors, ISBN-10/13, Publisher, Edition, Year, DDC (Dewey Decimal), Shelf/Rack Location, Total Copies.

FR-1.2: Auto-fetch bibliographic metadata via OpenLibrary / Google Books API upon entering ISBN.

FR-1.3: Individual copy tracking: Each copy has a unique Barcode and a bonded `rfid_tag_uid`.

FR-1.4: Tag Encoding Utility: Web interface to bind an unassigned RFID book sticker to a book copy with 1 click.

## Module 2: Patron & Identity Management

FR-2.1: User directory supporting Student ID, Department, Batch, Phone, Email, and status (Active/Suspended).

FR-2.2: Smart Card Binding: Ability to tap an ID card on the desk reader to associate its UID with the student profile.

FR-2.3: Borrowing Quota Rules:

Undergraduate: Max 4 books, 14 days loan period.

Postgraduate: Max 8 books, 30 days loan period.

Faculty: Max 15 books, 90 days loan period.

## Module 3: Circulation Engine

FR-3.1: Single/Multi-Book Issue: Patron card read + Book RFID tag(s) read -> validates limits -> writes loan records -> sets EAS security bit to `INACTIVE`.

FR-3.2: Instant Return: Book placed on return pad -> system detects book copy -> marks returned -> checks overdue fines -> sets EAS security bit to `ACTIVE`.

FR-3.3: Automated Fine Calculator: Daily cron or on-return trigger calculating fine based on customizable daily rate (e.g. ₹2/day after grace period).

## Module 4: Gate Access & Occupancy Tracking

FR-4.1: Tap-In Logic:

Checks if patron is already marked `INSIDE`. If not, creates `gate_sessions` record with `entry_time = NOW()`.

Increments live library occupancy counter.

FR-4.2: Tap-Out Logic:

Matches active open session, records `exit_time = NOW()`.

Computes total dwell duration in minutes (`exit_time - entry_time`).

Decrements live occupancy counter.

FR-4.3: Offline Resilience: If network drops, ESP32 buffers up to 1,000 taps in local SPIFFS flash with NTP timestamps and flushes upon reconnecting.

## Module 5: OPAC & Student Portal

FR-5.1: Responsive public search catalog with filters (Author, Category, Availability).

FR-5.2: Shelf locator indicator (e.g. "Rack B, Shelf 3").

FR-5.3: Student Dashboard: View current loans, due dates, calculated fines, and personal study hour logs.

## Module 6: Analytics & Administrative Reports

FR-6.1: Real-time occupancy gauge on Admin Dashboard (e.g. "68 / 150 Seats Occupied").

FR-6.2: Hourly and daily footfall graphs (peak study hours).

FR-6.3: Student study time leaderboard / monthly attendance report for academic credits.

FR-6.4: Overdue and lost book audit reports.

# 5. Database Schema Blueprint

erDiagram PATRONS ¦¦--o{ BORROW_RECORDS : makes PATRONS ¦¦--o{ GATE_SESSIONS : records BOOKS ¦¦--o{ BOOK_COPIES : contains BOOK_COPIES ¦¦--o{ BORROW_RECORDS : loaned_via PATRONS { uuid id PK string student_id UK string full_name string email string phone string rfid_card_uid UK string role string status timestamp created_at } BOOKS { uuid id PK string isbn UK string title string authors string publisher string category string ddc_code string shelf_location } BOOK_COPIES { uuid id PK uuid book_id FK string barcode UK string rfid_tag_uid UK string status boolean eas_security_bit int copy_number } BORROW_RECORDS { uuid id PK uuid patron_id FK uuid copy_id FK timestamp issued_at timestamp due_date timestamp returned_at decimal fine_amount string status } GATE_SESSIONS { uuid id PK uuid patron_id FK string gate_device_id timestamp entry_time timestamp exit_time int duration_minutes string session_status }

# 6. API Specifications (Sample Endpoints)

## 6.1 Hardware Edge Endpoints

#### `POST /api/v1/gate/tap`

Payload sent by ESP32/Reader:

{ "device_id": "GATE_ENTRY_01", "card_uid": "E28011702000208169A1B2C3", "direction": "IN", "timestamp": 1728300000 }

Response:

{ "status": "SUCCESS", "action": "ACCESS_GRANTED", "patron": { "name": "Khagendra Singh", "student_id": "STU2026-081" }, "current_occupancy": 42 }

#### `POST /api/v1/circulation/scan-pad`

Payload sent when books are laid on RFID pad:

{ "reader_id": "DESK_PAD_01", "patron_card_uid": "E28011702000208169A1B2C3", "book_tag_uids": [ "E004010045612341", "E004010045612342" ], "action": "ISSUE" }

# 7. Implementation Roadmap & Milestones

| Milestone | Target Deliverable | Duration |
| --- | --- | --- |
| **Milestone 1** | Project setup (Next.js + Prisma + SQLite/PostgreSQL) & Database Migrations | Days 1–3 |
| **Milestone 2** | Cataloging & Patron Management Modules with Web UI | Days 4–7 |
| **Milestone 3** | Circulation Engine (Issue/Return logic, fine calculator) | Days 8–11 |
| **Milestone 4** | Gate In/Out API + Virtual Web Hardware Simulator (Test without physical reader) | Days 12–14 |
| **Milestone 5** | ESP32 Firmware & WebUSB/Serial RFID Desk Reader Integration | Days 15–18 |
| **Milestone 6** | Real-time Occupancy Dashboard, Analytics & Reporting | Days 19–21 |

# 8. Success Metrics (KPIs)

Gate Transit Speed: Card tap to visual confirmation under 400 ms.

Circulation Throughput: 5 stacked books checked out in under 3 seconds (vs 45 seconds manual barcode scan).

Inventory Accuracy: Real-time visibility of book availability across all racks.

Attendance Completeness: 100% automated logging of student visits without human registers or intrusive AI surveillance.